Bioremediation- Environmental Biotechnology

Definition

Environmental Biotechnology is the application of biological processes and organisms—such as microorganisms, plants, and enzymes—to protect, restore, or sustainably manage the environment. It focuses on using biology-based techniques to treat wastewater, manage solid and hazardous waste, remediate polluted soils and groundwater, reduce air pollution, recycle resources, and support sustainable industrial practices.

### Significance of Environmental Biotechnology

### Environmental biotechnology is crucial because it:

- Protects and Restores the Environment

  - Treats wastewater and industrial effluents

  - Degrades hazardous pollutants and toxic chemicals

  - Cleans contaminated soil, groundwater, and marine environments

- Promotes Sustainable Resource Management

  - Converts waste into useful products (bioenergy, compost, bioplastics)

  - Supports recycling and waste minimization

  - Enhances soil fertility through bio fertilizers and bio pesticides

- Supports Public Health

  - Reduces disease-causing contaminants

  - Ensures cleaner water, air, and safer food environments

- Contributes to Climate Change Mitigation

  - Carbon sequestration and greenhouse gas reduction

  - Development of eco-friendly industrial processes

- Economic and Technological Benefits

  - Cost-effective and eco-friendly alternatives to chemical methods

  - Supports green industries and environmental policies

### Bioremediation-Use of microbes to degrade pollutants.

Bio augmentation is an environmental clean-up technique involving the addition of specific, cultured microorganisms (bacteria, fungi, or archaea) to contaminated soil or water to accelerate the degradation of pollutants, particularly recalcitrant organic compounds. It is commonly used in industrial/municipal wastewater treatment, bioreactors, and in-situ bioremediation.

Key Aspects of Bio augmentation:

- Process: It accelerates natural degradation, especially when native microbes are absent or inefficient, by introducing specialized, often genetically modified, organisms.

- Applications: It is primarily used for degrading chlorinated ethane’s, Petroleum Hydrocarbons (fuel, oil), and complex industrial waste.

- Wastewater Treatment: Utilized to restart activated sludge bioreactors.

- Factors Affecting Success:

  - Biotic: Competition with indigenous flora.

  - Abiotic: Environmental conditions like pH, oxygen levels, and nutrient availability.

- Advantages: Cost-effective, environmentally friendly, and efficient for specific, tough contaminants.

- Limitations: Difficult to monitor, potential for, low growth or "washout" of introduced bacteria, and the need for high inoculum levels (typically 106-107cells/mL).

- Bio augmentation: Adding selected microbes

- Bio stimulation: Enhancing native microbial activity

Biostimulation is a bioremediation strategy that enhances the natural degradation of contaminants by modifying the environment to stimulate existing indigenous (native) microorganisms. Unlike bioaugmentation, which introduces new foreign microbes, biostimulation focuses on making the current habitat more suitable for the local "cleanup crew" already present.

Core Mechanism

The process typically involves adding rate-limiting nutrients or substances that the local bacteria need but lack in sufficient quantities.

- Nutrients: Addition of nitrogen (N) and phosphorus (P) is standard, often in a C:N:P ratio of approximately 100:10:1 to 100:10:2.

- Electron Acceptors: Injecting oxygen (for aerobic processes) or nitrates and sulfates (for anaerobic processes) to fuel microbial respiration.

- Secondary Substrates: Adding easily degradable carbon sources like molasses, acetate, or lactate to support biomass growth that can then co-metabolize complex pollutants.

- Environmental Adjustments: Modifying pH (optimally 6.5–8.0) or moisture levels to create ideal conditions for growth.

Common Applications

- Oil Spills: Fertilisers are added to shorelines or open water to speed up the breakdown of petroleum hydrocarbons.

- Groundwater Cleanup: Injecting air (biosparging) or liquid nutrients into aquifers to remediate chlorinated solvents.

- Agricultural Soil: Adding organic matter (like compost or manure) to stimulate the degradation of pesticides and herbicides.

Pros and Cons

| Advantages | Disadvantages / Challenges |
| --- | --- |
| Cost-Effective: No need to cultivate or transport expensive laboratory microbes. | Site-Specific: Effectiveness depends heavily on local geology and existing microbial populations. |
| Eco-Friendly: Uses native species, avoiding the risks of introducing invasive or non-native organisms. | Nutrient Delivery: Tight soils (like clay) can prevent the even distribution of added nutrients. |
| Well-Adapted: Native microbes are already acclimated to the local environment and the contaminant. | Unwanted Growth: Risk of "mistargeting" where non-degrading or pathogenic bacteria consume the nutrients instead. |

- Phytoremediation: Plants absorbing heavy metals and contaminants

Phytoremediation is an eco-friendly bioremediation process that utilizes living plants to remove, transfer, stabilize, or destroy contaminants in soil, water, and air. Derived from the Greek phyto (plant) and Latin remedium (restoring balance), this "green technology" is a cost-effective alternative to traditional mechanical or chemical cleanup methods.

Core Mechanisms of Phytoremediation

The effectiveness of phytoremediation depends on the specific plant species, the type of contaminant, and the medium (soil or water) being treated.

- Phytoextraction (Phytoaccumulation): Plants absorb contaminants (mostly heavy metals) through their roots and translocate them to harvestable parts like leaves and shoots.

- Phytostabilization: Plants immobilize pollutants in the soil or groundwater, preventing their migration via erosion or leaching into deeper aquifers.

- Phytodegradation (Phytotransformation): Plants break down organic pollutants (like pesticides or solvents) into less harmful substances through internal metabolic processes or enzymes secreted by roots.

- Rhizofiltration: This technique uses plant roots to filter, absorb, or precipitate contaminants from contaminated water streams or groundwater.

- Phytovolatilization: Plants take up pollutants and release them into the atmosphere as volatile, less toxic gases through transpiration.

- Rhizodegradation (Phytostimulation): Root exudates (sugars, acids) stimulate the growth of soil microorganisms (bacteria/fungi), which then break down organic contaminants in the rhizosphere.

Key Plant Examples (Hyperaccumulators)

"Hyperaccumulators" are plants that can store exceptionally high levels of metals (100x more than common plants) without showing toxic symptoms.

| Plant Species | Target Contaminants | Application Example |
| --- | --- | --- |
| Sunflower (Helianthus annuus) | Pb, As, Cd, U, Cs | Cleaned radioactive water at Chernobyl. |
| Indian Mustard (Brassica juncea) | Pb, Cd, Ni, Zn, Se | Effective for heavy metal removal in industrial sites. |
| Poplar & Willow Trees | VOCs, TCE, Cd, Zn | Deep roots for groundwater cleanup and hydraulic control. |
| Water Hyacinth (Eichhornia crassipes) | Cu, Pb, Fe, Cr | Treats industrial wastewater and nutrient pollution. |
| Chinese Brake Fern (Pteris vittata) | Arsenic (As) | Exceptional ability to accumulate and volatilize arsenic. |

Pros and Cons of the Technology

- Advantages:

  - Cost-Effective: Up to 10x cheaper than traditional methods.

  - Sustainable: Solar-powered and improves soil fertility/health.

  - Aesthetic: Creates green spaces and restores ecosystems in situ (on-site).

- Limitations:

  - Time-Consuming: Can take several growing seasons (years) for significant results.

  - Depth-Limited: Only effective within the reach of the root zone.

  - Biomass Disposal: Harvested plants are often hazardous and require careful incineration or disposal.

Mycoremediation: Use of fungi for degradation

Mycoremediation is a branch of bioremediation that uses fungi to degrade, sequester, or transform environmental pollutants into non-toxic forms. Fungi are uniquely effective because their primary ecological role is decomposition, allowing them to break down complex molecules that other microorganisms often cannot.

Core Mechanisms of Degradation

Fungi do not simply "eat" toxins; they use sophisticated chemical processes to neutralize them:

- Enzymatic Degradation: Fungi secrete powerful extracellular enzymes (like laccase, lignin peroxidase, and manganese peroxidase) that break the strong carbon-hydrogen bonds found in organic pollutants.

- Biosorption: Pollutants, especially heavy metals, bind to the fungal cell walls (chitin and glucans). This immobilizes the toxins so they can be physically removed by harvesting the mushrooms.

- Bioaccumulation: Fungi actively absorb and store contaminants within their internal cellular structures.

- Mycofiltration: Using fungal mycelium as a "living filter" to capture and digest pathogens (like E. coli) and chemical runoff from water.

Key Fungal Groups & Species

Different fungi target specific pollutants based on their enzymatic profiles:

- White-Rot Fungi: The most studied group for mycoremediation. Species like Phanerochaete chrysosporium and Trametes versicolor are highly effective against petroleum hydrocarbons, pesticides, and industrial dyes.

- Brown-Rot Fungi: Focus on cellulose degradation and are useful for mineralizing synthetic chemicals.

- Molds (Aspergillus, Penicillium): Excellent for removing heavy metals (lead, cadmium, mercury) through biosorption.

- Oyster Mushrooms (Pleurotus ostreatus): Famous for breaking down DDT (80% in 28 days) and oil spills.

Comparison: Mycoremediation vs. Other Methods

| Feature | Mycoremediation | Bacterial Remediation | Chemical Cleanup |
| --- | --- | --- | --- |
| Approach | Natural / Low-tech | Microbial | Industrial |
| Cost | Low | Moderate | High |
| Reach | Extensive (mycelial network) | Surface-level | Targeted |
| Byproducts | Enzymes/Medicinal mushrooms | Variable | Often toxic waste |

Limitations

- Environmental Sensitivity: Requires specific pH, moisture, and temperature ranges to thrive.

- Speed: Generally slower than chemical incineration or mechanical removal.

- Toxicity Levels: If concentrations are too high, the pollutants may kill the fungi before they can start degrading them.

Fungal structures, primarily mycelium (the network of thread-like hyphae) and their secreted enzymes, are highly effective for pollution control—a process known as mycoremediation. Their unique, filamentous, and penetrating structure allows them to break down complex, persistent environmental toxins (such as plastics, hydrocarbons, and dyes) into harmless, simple molecules, making them superior to many bacteria in treating solid, recalcitrant waste.

ScienceDirect.com +2

Here is how specific fungal structures are effective for pollution control:

1. Mycelium: A Networked "Self-Healing Filter"

- Extensive Penetration: Mycelium grows as an extensive, dense network of hyphae that can penetrate deeply into soil, sediment, or contaminated materials, making it easier to reach pollutants that are not easily accessible to bacteria.

- Active Absorption (Biosorption/Bioaccumulation): The fungal cell wall acts as a passive filter (biosorption) due to the presence of functional groups like chitin, glucans, and proteins that bind with heavy metals such as lead, cadmium, and mercury.

- Mycofiltration: Fungal mycelium can act as a natural filter, trapping pathogenic microorganisms and suspended solids in polluted water.

ScienceDirect.com +3

2. Extracellular Enzymes: The Degradation Machinery

Fungi secrete non-specific extracellular enzymes that break down tough, complex compounds outside the cell.

Springer Nature Link

- Ligninolytic Enzymes (White-Rot Fungi): Species like Phanerochaete chrysosporium and Pleurotus ostreatus secrete powerful enzymes such as laccase, manganese peroxidase, and lignin peroxidase.

- Degradation Mechanism: These enzymes are "non-specific," meaning they can attack a wide variety of pollutants, including polycyclic aromatic hydrocarbons (PAHs), synthetic dyes, pharmaceuticals, and pesticides.

- Co-metabolism: As these enzymes decompose woody material (their natural food source), they incidentally break down pollutants without needing to metabolize the pollutant itself for energy.

Springer Nature Link +2

3. Fungal Secretions: Acids and Surfactants

- Organic Acids: Fungi (e.g., Aspergillus niger) secrete acids like citric acid, which solubilize heavy metals, converting them into less toxic forms that can be immobilized or removed from the environment.

- Biosurfactants: Some fungi produce surfactants that increase the bioavailability of hydrophobic contaminants like oil spills, making them easier to degrade.

ScienceDirect.com +1

Key Applications of Fungal Structure in Pollution Control

- Soil Remediation: Fungi decompose oil spills, pesticides, and PAHs in soil.

- Water Treatment: Mycelium filters out heavy metals and dyes from industrial wastewater.

- Plastic Degradation: White-rot fungi have been shown to break down plastic polymers (e.g., PE, PVC) by using their enzymes to rupture the material.

- Fungal Fuel Cells (FFCs): Fungi are used at the anode of fuel cells, breaking down waste materials (like sugar industry waste) and releasing electrons, simultaneously cleaning water and generating electricity.

ScienceDirect.com +4

Advantages

- Eco-friendly and Low-cost: It is a natural, sustainable method with low energy input.

- High Adaptability: Fungi can grow in extreme conditions (high acidity, temperature) where bacteria cannot.

- Dual Function: Fungi can both degrade organic pollutants and immobilize inorganic pollutants (heavy metals) simultaneously.

Applications

Oil spill clean-up

Bioremediation uses specialized "oil-eating" bacteria to break down complex hydrocarbons into harmless substances like water and carbon dioxide. This eco-friendly process is often preferred over chemical methods because it minimizes secondary pollution.

Key Oil-Degrading Bacteria

Over 70 to 100 genera of bacteria are known to have oil-degrading capabilities.

- Alcanivorax borkumensis: A "specialist" marine bacterium that thrives after spills and is highly efficient at degrading alkanes.

- Pseudomonas putida: Known as the first genetically engineered organism patented (by Dr. Ananda Chakrabarty) to consume multiple types of hydrocarbons.

- Pseudomonas aeruginosa: Frequently used due to its ability to produce bio surfactants, which emulsify oil and make it easier for other microbes to access.

- Cycloclasticus: Specialists that often follow generalist bacteria to break down more complex aromatic hydrocarbons.

- Bacillus subtilis: A common soil bacterium often used in consortia to enhance the overall degradation rate.

Heavy metal removal (Pb, Hg, Cd):

Bacteria detoxify and remove heavy metals through several distinct mechanisms of action. Since heavy metals are non-biodegradable, these processes focus on transformation (changing the metal's state/toxicity), sequestration (trapping the metal), or efflux (pumping the metal out).

1. Sequestration and Surface Binding

Bacteria use their structural components to trap metals and prevent them from entering the cytoplasm or to neutralize them once inside.

- Bio sorption: A passive, metabolism-independent process where heavy metal ions bind to functional groups (e.g., amino, carboxyl, phosphate, or hydroxyl) on the cell wall.

- Extracellular Sequestration: Bacteria secrete extracellular polymeric substances (EPS) or biofilms that act as barriers, accumulating metal ions outside the cell to protect the internal machinery.

- Intracellular Sequestration: Inside the cell, specialized low-molecular-weight proteins like metallothioneins or molecules like glutathione bind to metals to neutralize their toxicity.

2. Biotransformation (Redox Reactions)

Bacteria can change the oxidation state of metals, often converting a highly toxic or soluble form into a less toxic or insoluble one.

- Oxidation/Reduction: For example, bacteria like Bacillus sp. use mercuric reductase to reduce toxic Hg (II) to volatile Hg (0), which then diffuses out of the cell. Similarly, Micrococcus sp. can oxidize toxic As (III) to the less toxic and less soluble As (V).

- Bio methylation: Some bacteria add methyl groups to metals (like arsenic or mercury), which can make them more volatile and easier to remove from the immediate environment.

3. Bio mineralization and Precipitation

This mechanism involves converting free metal ions into stable, insoluble mineral salts.

- Carbonate Precipitation: Urease-producing bacteria (UPB) hydrolyse urea to produce carbonate ions (CO32−), which react with metals like Pb, Cd, and Zn to form insoluble carbonates (e.g., 𝑃𝑏𝐶𝑂3).

- Phosphate Precipitation: Phosphate-solubilizing bacteria (PSB) release organic acids or enzymes that free phosphate ions, which then bind with heavy metals to form stable phosphate minerals.

4. Active Transport (Efflux Systems)

To maintain internal homeostasis, bacteria use specialized energy-dependent "pumps" to expel toxic metal ions from the cytoplasm.

- Efflux Pumps: Proteins such as P-type ATPases, ABC transporters, and cation diffusion facilitators actively transport ions like Cu2+, Zn2+, and As3+ out of the cell.

- Arsenic Efflux: A common example is the Acr3p antiporter, which specifically facilitates the expulsion of trivalent arsenic.

5. Bioaccumulation

Unlike bio sorption, bioaccumulation is a metabolically active process where metals are transported into the cell (often via nutrient transport pathways) and stored in intracellular compartments or vacuoles. This is often used by bacteria to recover and stabilize metals from wastewater.

Pesticide degradation

Microbial degradation of pesticides is a complex biological process where microorganisms (bacteria, fungi, and algae) break down toxic chemical compounds into simpler, less harmful, or non-toxic substances. This mechanism is essentially a three-phase detoxification process:

1. The Three-Phase Metabolic Pathway

- Phase I: Activation: Microorganisms transform the parent pesticide through oxidation, reduction, or hydrolysis. This step introduces or exposes functional groups, making the compound more water-soluble and generally less toxic.

- Phase II: Conjugation: The transformed metabolites are joined (conjugated) with polar molecules like sugars or amino acids. This further increases solubility and decreases toxicity.

- Phase III: Compartmentalization/Mineralization: The conjugates are converted into secondary, non-toxic products. In the case of mineralization, the pesticide is completely broken down into inorganic substances like CO2 and H2O.

2. Primary Enzymatic Mechanisms

Microbes secrete specific enzymes to catalyse these reactions:

- Hydrolases: Cleave chemical bonds (esters, amides) by adding water. These are vital for degrading organophosphates and carbamates.

- Oxidoreductases: Facilitate electron transfer. Examples include laccases and peroxidases (often from white-rot fungi) which break down complex rings like those in DDT.

- Oxygenase’s (Mono- and Dioxygenases): Incorporate oxygen atoms into the molecule to break stable aromatic rings.

3. Modes of Microbial Interaction

- Direct Metabolism: Microbes use the pesticide as a primary source of carbon, nitrogen, or energy for growth.

- Co-metabolism: The pesticide is transformed incidentally while the microbe is metabolizing a different growth substrate. The pesticide itself provides no energy but is altered by existing microbial enzymes.

- Bio sorption: A metabolism-independent process where pesticides simply bind to the microbial cell wall through physical or chemical attraction.

Conclusion

- Bioremediation is an eco-friendly and sustainable approach for the removal of pollutants from the environment using natural biological agents. It operates through the interaction of three key components— pollutants, organisms, and environmental conditions —making it a “triple-corner” process.

- Microorganisms, plants, and enzymes play a crucial role in breaking down both organic and inorganic contaminants into less harmful or non-toxic substances. The efficiency of this process depends on factors such as temperature, pH, oxygen availability, and nutrient levels in soil, water, and air .

- Bioremediation offers several advantages, including cost-effectiveness, minimal environmental disturbance, and long-term sustainability. However, its success may be limited by slow processing rates and site-specific conditions.

- Overall, bioremediation represents a promising green technology for environmental management and pollution control, contributing significantly to ecological restoration and sustainable development.

## 9. References

- Alexander, M. (1999). Biodegradation and Bioremediation . Academic Press, New York.

- Pepper, IL, Gerba, CP, & Gentry, TJ (2015). Environmental Microbiology (3rd ed.). Academic Press.

- Madigan, MT, Bender, KS, Buckley, DH, Sattley, WM, & Stahl, DA (2021). Brock Biology of Microorganisms (16th ed.). Pearson Education.

- Singh, RL, & Singh, PK (2017). Principles and Applications of Environmental Biotechnology . Springer.

- Timmis, KN, et al. (2017). "The contribution of microbial biotechnology to sustainable development goals." Microbial Biotechnology , 10(5), 984–987.

- Pandey, A., Negi, S., & Soccol, CR (2016). Current Developments in Biotechnology and Bioengineering . Elsevier.